Procedure for Programming FWP to access GPRS

The following procedure may be followed to configure GPRS in FWP instruments:

- Install FWP CD in Computer

- Connect USB Cable to PC from FWP

- Go to My computer – properties- Hardware - device Manager - ports

- Select the profile USB to serial com port Com-B

- Go to Start – Settings – Control Panel – Phone and Modem – Modem – add

- Check ‘Don’t detect my modem’

- Next - Right click

- Select std 33600 bps modem, Select Com-B port

- Finish

This will install modem successfully

- Go to Modem

- Select Com B – properties – advanced

- Type AT+CGDCont=1,”IP”,”bsnlnet”

- Click change, default preference – Flow control

- Give as None – OK

- Right click my network places – Properties – Create new connection – next

- Connect to internet -Set up manually – Connect using a dial up modem

- Select std modem 33600 mbps - Next

- Give ISP name as “bsnl”

- Next – phone as *99#

- username and password  are left blank

- Add short cut – Click (One dial up icon will open)

- Click Icon – properties – select Modem 33600 then configure

- Enable hardware and block control