GENERAL TIPS FOR ENERGY CONSERVATION

Lighting:-

- Switch off all loads and check the meter if it is still moving, there could be wiring fault, it may be rectified to avoid further losses.

- Check for leakage loss in wiring and appliances.

- Keep clean all the lamp fixtures.

- Excess lighting/ Decorative lighting shall be avoided.

- Use CFL lamp for lighting Corridors, Entrance lobbies, toilets, and other common areas.

- Avoid group switching of lights for Offices.

- Use Electronic chokes and Energy efficient T5 or Slimline tubes.

Air-Conditioning:-

- Avoid direct sunlight in conditioned areas.

- North or South side walls are preferable for room to be air-conditioned.

- Check thermostats in all AC units for its effectiveness and performance. Setting shall be at 23 deg C.

- Check conditioned air leakages and arrest them.

- Replace life-expired ac units with energy efficient ac units.

Pumping system and other appliances:-

Water Pumps:-

- Use proper specification of motor/pump based on building height and tank size.

- Replace old pumps with new and more efficient pumps.

- Check and correct leaking lines and taps.

Water Heater:-

- Ensure good insulation of tank and hot water line.

- Thermostat setting at 50-55 deg.C

- Provision of non return valve in cold water line to be ensured.