Annexure-III

## CREATION of PRI in EWSD Switch

## Necessary HW reqd. in EWSD switch

## LTGN for PA

### LTGN for remote  PA (+ PCM30 trunks)

HW equipment:	M:GPNYD (32 MByte)

F:LTGN(A);

Interface:	4 x 2 Mbit/s with 30 channels each

Applications: Connection of PA,

SW:	B load type, i.e. subscriber load type

## LTGP for PA

### LTGP for remote  PA (+ PCM30 trunks)

HW equipment:	M:GPPYG

F:LTGP(A);

Applications: Connection of  PA,

SW:	B load type, i.e. subscriber load type

# Sample Database for ISDN PRI as DDI

CR LTG : LTG = TSG-LTG, TYPE = LTGB, LDPARP = 12;

CR LTU: LTG=tsg-ltg, LTU= , TYPE= D30, APPLIC= CCSPA ;

CR PA: PA= PAname, TYPE= FIXED, LTG=tsg-ltg, LTUP=<LTU POSITION> ;

CR DN : LAC = XXX, DN = 6772000 && 6772999 ;

CR PBX: DN=6772xxx , LAC= , SERV= , OPMODE=iic&iog , COS=ddi , HUNT= ..,minmax = ….., DINO = …., SSDI = …..,OPTRCL=;

CRPBXLN: LAC= , DN= , EQN=tsg-ltg-ltu-16 (signaling channel), LNO= , CAT= IPADCH, OPMODE= IIC ;

CRPBXLN: LAC= , DN= , EQN=tsg-ltg-ltu-channel, LNO= ,  CAT= IPABCH,  OPMODE=iBW ,OPTRCL=;

For more details please refer to the manual OMN : EXCH.

# Sample Database for ISDN PRI with INDIVIDUAL TIME SLOTS

CR LTG : LTG = TSG-LTG, TYPE = LTGB, LDPARP = 12;

CR LTU: LTG=tsg-ltg, LTU= , TYPE= D30, APPLIC= CCSPA ;

CR PA: PA= PAname, TYPE= FIXED, LTG=tsg-ltg, LTUP=<LTU POSITION> ;

CR DN : LAC = XXX, DN = 6772000 && 6772999 ;

CR PBX: DN=6772 , LAC= , SERV= , OPMODE=iic&iog , COS=ddi , HUNT= ..,minmax = ….., DINO = …., SSDI = …..,OPTRCL=;

CRPBXLN: LAC= , DN= , EQN=tsg-ltg-ltu-16 (signaling channel), LNO= , CAT= IPADCH, OPMODE= IIC, lnatt=mpn ;

CRPBXLN: LAC= , DN= , EQN=tsg-ltg-ltu-channel, LNO= ,  CAT= IPABCH,  OPMODE=iBW ,OPTRCL=;